Name______________________________________Class:_________________Date:____/____/____

Inductance

Lab Activity #1 – Determine Transformer Ratios

Equipment and materials

- Filament transformer (step down approximately 110v to 6v)

- Autotransformer or signal generator

- 30-ohm, 5W resistor

- Multimeter

- Electrical wire to build circuit

Procedure

1.  Construct the following circuit (Figure 1)

- Adjust the signal input to 100 volts

- Close the switch and record the secondary current, (IS)

- Measure and record the primary voltage, (VP) _______ _____

- Measure and record the secondary voltage, (VS) ______ ____

- Measure and record the load resistor (approximately 30 ohms) _______

- Compare the measured secondary current (IS), with that obtained using Ohm’s law (VS/RL), and explain the difference ____________________________________________________________________________________

- Measure and record (IP) _____________ _____

- Calculate the turns ratio of the transformer ______________________________________

____________________________________________________________________________________

- Remove the load resistor and leave the secondary open

- Measure and record the primary current (IP), with an open secondary

_________________ _____

- Compare your voltage ratio with that indicated by this transformer manufacturer

- Turn off circuit, disconnect components, and place materials in the proper storage area